BRANDON VO

Due F 11/10

The description below will be updated with additional information so check this page regularly.

Project specifications:

- Objective: measure and display the period and duty cycle of a square signal connected to the AVR microcontroller.

- Input signal can be as low as 1mHz and as high as 100MHZ. Input duty cycles vary from 0% to 100% in increments of 1%.

- The two measurements must be displayed on the LCD screen concurrently. The period must be displayed in seconds with units "s" at the end. Use "%" for the duty cycle measurement. For the period you may use "u" or "m" for micro or milli.

- The LCD 4-bit mode is preferred, but 8-bit mode is OK.

- Measurements must be continuous: this means that measurements are constantly being updated, such that if the input changes, the measurements will automatically be updated.

- You should build your project in Simulation IDE and use Microchip Studio for programming.

- Use the function/clock generator and the oscilloscope in Simulation IDE to test your design.

Process:

- Setup the Timer ICP

- Detect 2 consecutive positive edges (or negative edges) for a period count.

- Detect a positive and a negative edge for duty cycle count.

- Convert period count to seconds and convert the duty to a percentage.

- Convert numbers (period and duty cycle) to strings

- Setup the LCD and display the strings.

- Repeat step 2, infinitely.

Algorithms:

1. Setup the Timer ICP:

- Consult the notes on how to setup ICP on a 16-bit Timer.

2. Detect 2 consecutive edges for a period count:

- Consult the notes on how to detect the 2 consecutive rising edge for period calculation.

- The difficulty lies in setting the prescaler. There are 2 options for the prescaler: (a) Set the prescaler to 1 and rely on the overflow for calculations
(b) dynamic (changing) prescaler, that changes as needed. (Hint: decrease the prescaler if there's an overflow).

- It is advisable to use long int variable for the period and overflows to help with with calculations.

4. Convert period count to seconds and convert the duty to a percentage:

- Period (s) = (Period count/16MHz) * prescaler

- Duty % = (duty count/period count) * 100

- Use long int variables definition for Period and Duty

- Perform the multiplication BEFORE division (100/1000 = 0 as an integer!)

5. Convert number to strings:

- The numbers are the period and duty cycle calculations.

- Use successive division and modulus to isolate each digit, e.g.

  - num = 1234

  - d1 = num%10 = 4, d2 = num/10%10 = 3, d3 = num/100%10=2, ...

  - Convert each digit to ASCII (add 0x30) and store in a string

Submission:

Submit 1 zip file that contains

- a report document,

- a simulation IDE file (.sim)

- a C program (.C)

- a hex file for the simulation (.hex)

Report:

In your report include the following (don't do a full report format):

- Clearly state:

  - if your code use overflow to detect low varying signals

  - automatically changes the prescaler as needed.

- Schematic showing all components and their pin connections (ie screenshot you simlation IDE). The circuit may be too big to fit on a page, so split it into separate images. Label wires if needed (e.g if they're cut-off).

- Derive:

  - MAX frequency that can be detected accurately.

  - MIN frequency that can be detected accurately.

  - The max/min duty cycle that can be detected at the max frequency

  - The max frequency at which duty cycle is properly detected in 10% increments

  - The max frequency at which duty cycle is properly detected in 1% increments.

- Code:

  - Code will not be graded if you don't follow instruction below:

    - INDENT code

    - DON'T double space the code or use a font larger that 11pts

    - DON'T use running comment lines (that continue on the next line)

- Test results (image screenshots):

  - MAX freq detected properly

  - MIN freq detected properly

  - MAX freq at which 1%, 99% duty is captured and detected properly

  - MAX freq at which 5%, 95% duty is captured and detected properly

  - MAX freq at which 10%, 90% duty is captured and detected properly

  - Screenshot should clearly show the settings on the input and output of the scope.

Project 3 Report

Explanation:
Code uses overflow, does not change prescaler

Circuit Diagram

Max Freq Detected Accurately ( ~25 KHz, 50% duty)

Min Freq Detected (~200 Hz, 50% duty)

Max Freq Detected with 99% Duty (~2 KHz)

Max Freq Detected with 95% Duty (~12 KHz)

Max Freq Detected with 90% Duty (~25 KHz)

Code

// Project 3

#define F_CPU 16000000L //16MHz

#include <avr/io.h>

#include <util/delay.h>

#include <stdint.h>

#include <avr/interrupt.h>

volatile uint16_t tovCnt = 0;	// use volatile for interrupt variables

void setupLCD(void)

{

DDRF = 0xFF;            //LCD 8bit data

DDRK = 0x03;            //PTK0= En, PTK1=RS

char2LCD(0x38, 0);      // set 8-bit data, 2-line display, 5x8 font

char2LCD(0x0F, 0);      // turn on display, cursor, blinking

char2LCD(0x06, 0);      // move cursor right

char2LCD(0x01, 0);      // clear screen, move cursor to home

}

void char2LCD (unsigned char data, unsigned char IR_DR)

{   PORTK = IR_DR;            // set instr or character

PORTK = IR_DR|0x01;        // pull E signal to high

PORTF = data;

PORTK = IR_DR;

}

void str2LCD (unsigned char *ptr)

{

while (*ptr)                 //ascii-Z strings (null-terminated)

{

char2LCD(*ptr, 2);

// _delay_ms(500);

ptr++;

}

}

// Inputs a char at a specific area of a string to the LED

void input_char(int8_t character, uint8_t *str, int8_t len)

{

int8_t i;

for (i = len - 1; i >= 0; i--){

str[i] = character;

}

}

// Converts stuff into arrays/strings for LCD output

void output_LCD(uint32_t num, uint8_t *str, int8_t len)

{

int8_t i;

for (i = len -1; i >= 0; i--){

str[i] = (num % 10) + 0x30;

num /= 10;

}

}

ISR (TIMER4_OVF_vect) {

tovCnt++;

}

int main(void)

{

setupLCD();

uint16_t period, duty_cycle, time_start, time_end;

TCCR4A = 0;

DDRL &= 0XFE;

TIFR4 = 0X2F;

TIMSK4 = 0X01;

sei();

while(1)

{

char2LCD(0X01,0);

str2LCD("Period: ");

str2LCD("      us");

char2LCD(0XC0,0);

str2LCD("Duty Cycle:    %");

// Period

TCCR4B = 0X43;	// Rising Edge Flag

TIFR4 = 0X21;	// clear ICF4 and OVF

TCNT4 = 0;

tovCnt = 0;     // Reset overflow counter

while(!(TIFR4 & 0x20)); // 1st Rising Edge

uint16_t period_start = tovCnt * 65536 + ICR4;

TIFR4 = 0x21;	// clear ICF4 and OVF

tovCnt = 0;     // Reset overflow counter

while(!(TIFR4 & 0x20)); // 2nd Rising Edge

uint16_t period_end = tovCnt * 65536 + ICR4;

uint16_t period = period_end - period_start;

// Duty Cycle

TIFR4 = 0x21;

TCNT4 = 0;

tovCnt = 0; // Reset overflow counter

while(!(TIFR4 & 0x20)); // 1st Rising Edge

uint16_t time_start = tovCnt * 65536 + ICR4;

TIFR4 = 0x21; // clear ICF4 and OVF

tovCnt = 0;   // Reset overflow counter

TCCR4B = 0x03; // Falling Edge Flag

while(!(TIFR4 & 0x20)); // Falling Edge

uint16_t time_end = tovCnt * 65536 + ICR4;

uint16_t duty_cycle = time_end - time_start;

// Duty Cycle and Period

duty_cycle = (duty_cycle * 100) / period;

period = (period * 64) / 16;

output_LCD(period,"      us",5);

output_LCD(duty_cycle,"   %",3);

_delay_ms(300);

}

}